Adrienne Electronics Corporation        August 1995

INTRODUCTION:

This file provides documentation for the "TC2KEY" program,

which was written by Adrienne Electronics Corporation in July 1992

at the request of (and funded by) The MacArthur Foundation.

PURPOSE:

This program uses a list of time codes and corresponding keypresses,

along with an Adrienne Electronics Corporation AEC-BOX-80 series

VTR controller, to simulate keyboard presses at predetermined times.

The time reference signal is read from the VTR via the AEC-BOX-80

over a standard COM1/COM2 RS232 serial port.  Since this program is a TSR,

and thus operates transparently in the background, it can be used to provide

keyboard input to any other program, thus automating said program.

COMPUTER SYSTEM REQUIREMENTS:

This program has been designed to operate on generic IBM PC/XT/AT type

computers using DOS 2.0 or later operating systems.  A standard COM1 or

COM2 RS232 serial port, with hardware interrupt lines, is also required.

COMMAND LINE SYNTAX FOR TSR INSTALLATION:

The DOS command line should be something like "TC2KEY TEST1234.T2K 1".

The file whose name you provide in the command line holds the

ASCII list information which you have previously created.

The program will abort if you enter a file name which cannot be found,

or if you choose a file name whose extension is not "LST" or "T2K".

The final numeric digit (1 or 2) indicates which RS232 serial port

(COM1 or COM2) the AEC-BOX-80 is connected to.

LIST FILE FORMAT:

The program input must be a standard ASCII text file, such as that created

by the MKTC2KEY program.  The exact file format required is explained in

detail in the MKTC2KEY.DOC file.  If you have any questions about the file

format, it is best to create a simple file using the MKTC2KEY program,

then view its output using your favorite text editor (in NONDOCUMENT mode!).

TC2KEY PROGRAM OPERATIONS:

Once you have entered the DOS command line, the TC2KEY program first checks

the validity of the information you have provided.  The program will print

a short message and abort (with a nonzero return code for batch files)

if it detects a missing list file, incorrect file name, missing serial

port number, missing AEC-BOX-80, list file syntax error, etc..

Assuming that everything checks out OK, the program will install itself

into memory, then return to DOS (with a zero return code), and begin

providing keypress information to your application program, as determined

by the list file data.

COMMAND LINE SYNTAX FOR TSR REMOVAL:

The DOS command line should be "TC2KEY OFF".  This will remove the TSR

from memory, thus freeing up memory space, disabling further keypress

insertions, and making room for the next TC2KEY installation.

RECOMMENDED BATCH FILE USAGE:

The TC2KEY program can be automatically installed, used, then removed,

by using a simple batch file such as the following:

TC2KEY TEST1234.T2K 1

YOURPROG

TC2KEY OFF

Use of such a batch file is highly recommended, where possible, because

then you can't "forget" to remove the TSR from memory.  Saves time too.

RESTRICTIONS ON YOUR PROGRAM:

Once the TC2KEY program has been installed, it assumes that your program

is not sending ANY commands to the AEC-BOX-80.  Otherwise things won't

work properly.  However, it is OK to make manual keyboard entries while

the program is running, provided that you and the TC2KEY program don't

try to enter something at the same time.

NTSC/SMPTE VS. PAL/EBU OPERATIONS:

The TC2KEY program reads the current format from the AEC-BOX-80,

so the AEC-BOX-80 must be properly set up (using the BOXSETUP program)

ahead of time.  It is assumed that the time codes you have entered into

the list file are valid with the current AEC-BOX-80 format setup.

PROGRAM DISTRIBUTION:

This program is copyrighted by Adrienne Electronics Corporation.

Permission is hereby granted to copy and distribute this program

free of charge, provided that it is used only in conjunction with

our AEC-BOX-80 products.  Any other copies, modifications, or usages

are expressly forbidden under this agreement.  We have received only

a modest fee for development of this program, and receive no "per-copy"

income, so trust that you will honor this agreement.

IF YOU NEED HELP:

Questions and/or ideas for improvements should be sent to:

Adrienne Electronics Corporation

Attn: Bruce Waggoner

2196 E. Ford Avenue

Las Vegas, NV 89123

U.S.A.

tel: +1-702-896-1858

fax: +1-702-896-3034

EOF

#########################################