USE OF ADRIENNE ELECTRONICS CORPORATION

PC-LTC, PC-VITC, and PC-VLTC BOARDS

WITH COMPREHENSIVE VIDEO SUPPLY'S "LOG-IT PC" SOFTWARE

May 1991

The LOG-IT PC tape logging software from Comprehensive Video Supply

Corporation (CVSC) is designed to work with their model EL-TCR

LTC reader board.  However, the LOG-IT PC software can be used with

any of our time code boards instead by following these instructions:

1) Install our time code board as usual.

2) Install our LOGKEY program by running it (once only!).

3) Enter the LOG-IT program, and enter the LOGGING mode.

4) Press "F9" (as usual) to enter IN point data.

5) Press "F10" (as usual) to enter OUT point data.

6) When finished, run our NOLOGKEY program to remove it from memory.

7) Note that our LOG.BAT program does most of this for you.

When running on some of the older (slower) PC's, there is a noticeable

delay when entering time code data in this manner.  You will also note

that there is no "real time" time code display present.

For a better long term solution, you can either lobby CVSC to alter

their software to work better with our boards, or you can try to

convince us to write a similar (or better) tape logging program.

BEW

*** TRADEMARK NOTICES ***

Adrienne, AEC, PC-LTC, PC-VITC, and PC-VLTC are trademarks of

Adrienne Electronics Corporation.

LOG-IT PC is a trademark of Comprehensive Video Supply Corporation.

EOF

#########################################################################################################################