Adrienne Electronics Corporation      TC-CLOCK.DOC       10/04/95

LEGAL NOTICES:

1) Henceforth, "TC-CLOCK" is a trademark of Adrienne Electronics Corporation.

2) The TC-CLOCK program is copyrighted by Adrienne Electronics Corporation.

3) You are only authorized to use the TC-CLOCK program in its original form,

or as modified per the instructions below, and only in conjunction with

our time code boards.  Any unauthorized duplication, modifications,

or other use of this program can get you into big trouble.

PURPOSE:

1) This file documents the installation and use of the TC-CLOCK.COM program.

FUNCTION:

1) When properly installed, the TC-CLOCK program will force the BIOS and DOS

clocks in your computer to match the time code being read by our board,

with a maximum error of about 150ms.

COMPUTER REQUIREMENTS:

1) The TC-CLOCK program assumes that you are using one of our time code

reader boards in an IBM PC/XT/AT personal computer (or 100% compatible).

We do not guarantee operation on "clones" which really aren't the same.

2) DOS 2.00 or higher is required (if you can read this disk you're OK).

3) When installed, the TC-CLOCK program uses up about 1200 bytes of RAM.

INSTALLATION:

1) For a PC-LTC/IOR board at 2A0h, execute "TC-CLOCK IOR@2A0".

2) For a PC-LTC/MMR board at CC00h, execute "TC-CLOCK MMR@CC00".

3) For other time code boards, execute "TC-CLOCK xxx@yyyy", where xxx is

the board name (RDR, RG1, MSG, etc.), and where yyyy is the board address.

For example, execute "TC-CLOCK RDR@CC00" for a PC-LTC/RDR board at CC00h.

4) Change the command line examples above, as needed, if your board is

installed at some other address.

5) If you hear a beep while the program is being installed, something

must be wrong with the command line structure.

6) The only way to know if the program is working properly is to plug in

a time code source, then use the DOS "TIME" command to see if the

computer's time matches the time code input.

7) You may wish to include the installation line as part of a batch file,

such as the AUTOEXEC.BAT file in your root directory,

which is automatically executed every time you turn on (or reboot)

your computer.

HOW IT WORKS:

1) When installed, the program will "terminate and stay resident",

which means that the program remains in RAM and works in the background,

transparent (hopefully) to any other program(s) you are running.

2) About once each second, the TC-CLOCK program will force the DOS and BIOS

time-of-day clocks to match the time code input signal, but only if the

time difference is more than about 150ms.  Thus in most cases, where

the computer and time code input clocks have virtually the same frequency,

the time "jams" will happen infrequently (after the first one happens).

3) A software filter is included which causes the program to ignore

occasional "jumps" in the time code data, possibly because of read errors.

This prevents the DOS and BIOS clocks from "jumping" unneccesarily.

REMOVAL:

1) The TC-CLOCK program can be removed from RAM at any time, thereby

stopping the program and freeing up memory, by executing "TC-CLOCK OFF".

You may wish to include this line as part of a batch file.

NEW FEATURE:

1) Effective with TC-CLOCK revision "A2", you can force the TC-CLOCK

program to also update your computer's CMOS clock by modifying the

TC-CLOCK.COM file as follows (from the DOS prompt):

DEBUG TC-CLOCK.COM

-F 104 L1 01

-W

-Q

2) Now your computer will always boot up with (approximately) the correct

time, since the battery operated CMOS clock inside your computer

runs even when AC power to your computer is turned off.

3) This feature will obviously not work properly on older PC's which

have no CMOS clock, such as the original IBM PC and IBM PC/XT.

The TC-CLOCK program will still operate properly, but the computer's

clock will have to be re-set every time you turn on the computer.

THAT'S ALL!

####################################