Adrienne Electronics Corporation       VLTCKEY.DOC       10/04/95

LEGAL NOTICES:

1) The VLTCKEY.COM program is copyrighted by Adrienne Electronics Corporation.

2) You are only authorized to use the VLTCKEY program in its original form,

or as modified per the instructions below, and only in conjunction with

our time code boards.  Any unauthorized duplication, modifications,

or other use of this program can get you into big trouble.

3) PC-VLTC is a trademark of Adrienne Electronics Corporation.

PURPOSE:

1) This file documents the installation and use of the VLTCKEY.COM program.

FUNCTION:

1) When properly installed, a simple keystroke (Alt-Enter) will read time code

from your PC-VLTC/RDR/RG3/RG2/RG1 board into DOS programs, word processors,

a database, your application program, etc.

2) The Alt-Enter key combination is accomplished by holding down the Alt key

while you press and release the Enter key.

FORMAT:

1) The time code format is HH:MM:SS:FF, a field of 11 ASCII characters.

2) If something isn't working properly, you will see a ??:??:??:?? display.

COMPUTER REQUIREMENTS:

1) The VLTCKEY program assumes that you are using one of our PC-VLTC boards

in an IBM PC/XT/AT personal computer (or 100% compatible).

We do not guarantee operation on "clones" which really aren't the same.

2) DOS 2.00 or higher is required (if you can read this disk you're OK).

INSTALLATION:

1) Assuming that your PC-VLTC board is installed at address CC00H,

just run the VLTCKEY program once (type VLTCKEY, then the Enter key).

The program will now be installed (try it).

2) See the "CUSTOMIZING" section for board addresses other than CC00H.

3) Installing this program more than once will lead to multiple

time code entries per keystroke, and should be avoided.

4) This program occupies just under 1kbyte of RAM.

5) You may want to run this program automatically every time you turn

your computer on, by executing it from the AUTOEXEC.BAT file.

Just enter the single line command VLTCKEY, making sure to use

a CHDIR or CD command if necessary to point to the proper directory.

We prefer that you execute this program after all other such programs

have been executed (put it towards the end of the file).

6) The PC-VLTC board is assumed to be in a time code reader mode,

which it is every time you turn the computer on (by default).

So VLTCKEY requires no PC-VLTC board software to be written.

HOW IT WORKS:

1) When installed, the program will "terminate and stay resident",

which means that it is always standing by in RAM to do the work

when called for.

2) Any time the special Alt-Enter key combination is pressed,

the VLTCKEY program reads the current time code, then intercepts

all keyboard BIOS (16H) function calls and enters the time code for you.

3) As far as application programs can tell, you are entering

the time code manually via the keyboard.

CUSTOMIZING - GENERAL:

1) As detailed in the sections below, many aspects of the VLTCKEY program

can be customized for your particular needs.

2) Before doing any customizing, be sure to save a backup copy of VLTCKEY.COM!

You may want to copy it to a file called AECVLKEY.COM.

3) Customizing is accomplished by using the DEBUG program which came on

your DOS diskette to alter the VLTCKEY.COM file.

The following example changes the board address (byte 104H) to D400H (D4H):

a) DEBUG VLTCKEY.COM

b) -F 104 L 1 D4

c) -W

d) -Q

CUSTOMIZING - BOARD ADDRESS:

1) As shown in the example above, byte 104H (normally CCH) can be changed

to the upper byte of the PC-VLTC board segment address (C0,C4,....,D8,DC).

CUSTOMIZING - DISPLAY FORMAT:

1) By setting bit 0 high (01H) in byte 106H, the program will return only

8 characters (HH:MM:SS) instead of the usual 11 character display.

CUSTOMIZING - KEY COMBINATION:

1) Bytes 108H, 109H, and 10AH can be altered to use some key combination

other than Alt-Enter if needed.

2) Byte 108H is the mask for the shift key status byte.

3) Byte 109H is the data for the shift key status byte.

4) Byte 10AH is the scan code (not ASCII) for the key itself.

5) Since this can get complicated quickly, and since almost all other

key combinations are used by other programs, we strongly suggest

that you not change these codes unless absolutely necessary.

THAT'S ALL!

#######################################################################################################